iWiscloud® Infrared / Multi-frequency

Control Signal Transmitter (CST)

# http://en.iwiscloud.com

Warning:

Please do not attempt to dismantle this delicate product, willful manipulation might lead to electrical failure. Warranty will be void if product is found to be tampered with.

Disclaimer:

This is not a standalone product; It must be deployed with either iWiscloud Smart Home Control Center or Wisbox Mini.

There might be discrepancies with this manual and the product as the product is constantly being improved upon. For the latest manual please go to en.iwiscloud.com to download the latest product manual. This product is only to be used as intended any misuse will void the warranty of the product.

Both Multi Frequency Control Signal Transmitter (Infrared Signal, 315MHz, 433 MHz) and Infrared Control Signal Transmitter have similar functionalities and hereinafter generally referred to as CST (Control Signal Transmitter).

- Product Introduction

This product had been designed and manufactured in accordance to strict International safety standards and achieved CE certifications.

1.1 Features

- Built-in Infrared, 315MHz, 433MHz Transceiver Module, allows receiving and transmitting control signals.

- Each individual CST can pair with multiple devices, learn, mimic and store up to 200 control signals from existing remote controllers.

- Deployed together with either iWiscloud Smart Home Control Centre or Wisbox Mini, to achieve long distance remote control home appliance (Infrared/315&433MHz) via phone, tablet, or PC.

- With our propriety Nebula Protocol mesh extension capabilities, iWiscloud CSTs are able to receive control signals up to hundred floors apart.

1.2 Product Specifications

|  | Multi-frequency CST | Infrared CST |
| --- | --- | --- |
| Voltage | 100-240 V | DC 5V |
| Infrared Effective Range | ≤ 10 meters | ≤10 meters |
| Control Signal Storage | 200 | 200 |
| Maximum Tilt Level Angle | ≤120° | ≤120° |
| Maximum Vertical Angle | ≤60° | ≤60° |
| Recommended Installation Height | Between 5 ft. and 6.5 ft (1.5m ≤ h ≤ 2m) | Between 5 ft. and 6.5 ft (1.5m ≤ h ≤ 2m) |
| Operating Temperature | -5℃~ 50℃ | -5℃~ 50℃ |

  - Proposed Deployed Locations

CSTs are similar to fixed universal remote controllers that can be deployed anywhere within your home (e.g. living room, bedroom, car garage, basement storage, kitchen) where you can realize centralize control of your existing remote control home appliances or controls such as air conditioner/heater, fan, television, audio set, motorized control (windows, curtains, garage doors), etc.

2. Setting up

2.1 Installation Recommendations

Ensure the CST is located according to the figure below for the recommended position for installation. (Infrared signal controls the angle may be offset in position on the basis of this fixed adjustment).

Diagram 1 - Top View

Diagram 2 – Side View

Note:

● Please do not concurrently use other infrared device or wireless control while configuring the CST. This would help to prevent external interference.

● Please do not place the CST within an enclosed metal cover as this will affect its wireless communication capabilities.

2.2 Configuring Multiple CSTs

User may require deploy multiple CSTs within different rooms (e.g. Living room, main bedroom, guestroom, etc.). To achieve centralized controls of the CSTs via iWiscloud Smart Home Control Center or Wisbox Mini, it is required to distinguish the CSTs. CSTs are being distinguished by the jumper cap(s) [see below diagram] placement to signify different rooms.

Multi-Frequencies CST Jumper Cap Replacing

0）Living room   0000

1）Main bedroom   0001

2）Child room 0010

3）Guest room 0011

4）Study room   0100

- Toilet   0101

Infrared Transmitter Jumper Cap Replacing

0）Living room   0000

1）Main bedroom 0001

2）Child room 0010

3）Guest room 0011

4）Study room 0100

5）Toilet 0101

Please see below illustrations on the jumper cap(s) replacement configuration:

| Code | Room/ Area | Multi-frequency CST | Infrared CST |
| --- | --- | --- | --- |
| 0000 | 1 |  |  |
| 0001 | 2 |  |  |
| 0010 | 3 |  |  |
| 0011 | 4 |  |  |
| 0100 | 5 |  |  |
| 0101 | 6 |  |  |

Note:

● Without any jumper caps placed, the CST default configuration is for Room 1;

● For changing jumper caps settings, please ensure CST is electrical powered off;

● CST must be reset (see following paragraph) after jumper cap configuration changes;

● The position to place the jumper caps is different for both Infrared Transmitter and Multi Frequency Transmitter. The jumper caps will be placed in different positions for different room settings.

It is recommended to perform a CST factory reset for every new deployment. It is performed by pressing on the “Reset” button on the circuit board for continuous 3 beeps onwards.

2.3 Steps to Set-up CST

Pairing CST With iWiscloud Smart Home Control Center:

- Perform a factory reset of the IR device.

- On the iWiscloud control center, you’ll see the iWiscloud logo at the main screen’s top right corner.

- Click on the letter "W" in order to enter the pairing addressing, the main screen will display a pop-up box with the various rooms. This screen shows whether the pairing is successful or not.

- After successfully addressing, click "OK" to exit; then click the letter "d" of the iWiscloud’s logo, it then checks to confirm an Infrared / Multi Frequency Transmitter hosted a successful connection, then click "OK" to exit.
Note:

● If the connection is not successful then repeat the first three steps, check again. Continue to click on the “W” once again.

● When the connection is successful, you may confirm whether both connections are matched. There is no need to click on the "W", because the room corresponding to the Infrared / Multi Frequency Transmitter has been paired successfully. Confirm the connection by simply clicking on the "d" once again to check.

Pairing CST With Wisbox Mini

- Log in to the Wisbox Mini website and click the desired room where you would like to locate the IR repeater. Click on “Add device” to add an “IR repeater” and click save.

- Click the new IR repeater icon and click on “searching address” button to pair the IR repeater to the Wisbox Mini.

- Click on “checking” button to test if the pairing is successful. If successful, the Wisbox Mini should make a beep sound.

Pairing CST With Home Remote Controllers

(Please refer to iWiscloud Smart Home Center/Wisbox Mini operation manual for detailed steps)

- Identify the remote controller frequency that the electronic device to be pair is working on. For example, TV or audio set controllers are usually Infrared, Security devices such as door magnet or door bells are on 315Mhz and motorized windows or curtains are on 433Mhz. Ensure the electronic device is powered on.

- Activate one of the required "Infrared / 315MHz / 433MHz / Security device” pairing mode on iWiscloud Smart Home Center/Wisbox Mini

- Assuming electronic device is already present on iWiscloud Smart Home Centre/Wisbox Mini, please select the electronic device to be paired. The generic control options should appear.

- Select one of the remote control commands to be paired. The iWiscloud Smart Home Centre/Wisbox Mini would start beep continuous (for 10 seconds) to signify it is in  “pairing mode”. You should issue the command on existing remote controller towards the CST. If the pairing was successful, the iWiscloud Smart Home Centre/Wisbox Mini would give off a long beep (1 seconds long). Otherwise, it would continuous beeping till the 10 seconds period is over. Please retry this step if the pairing was unsuccessful.

- Repeat step 4 for all the remote control commands to be paired.

- Assuming the pairing was successful, please attempt to execute the “paired” command after deactivating “Infrared / 315MHz / 433MHz / Security device” pairing mode. Repeat step 4 if results not as expected.

Note:

● For air-condition remote control performed by CST, by default, the “powered on” command is always accompanied with “temperature setting” command. Hence, it is recommended for users to pair the “up/down temperature” commands accordingly to ensure the desired temperature is set.

3. Product Checklist

| Items | Quantity |
| --- | --- |
| iWiscloud® Infrared / Multi-frequency Control Signal Transmitter | 1 |
| Operating Manual | 1 |
| Jumper Caps | 5 |
| Fastening Screws | 3 |
| Warranty Card | 1 |
| Product QC Certificate | 1 |

4. Customer Service and Warranty Coverage

Wiscloud  provides the following International warranty coverage for iWiscloud Infrared / Multi Frequency Transmitter:

| Warranty Period | 12 months |
| --- | --- |
| Scope of Warranty | Electronic parts & sensory module |
| Warranty Service | Carry in - one to one product exchange |

1.	If there is performance failure, customers can return, exchange or repair products within 7 days from the date of original retail purchase.

2.	If there is performance failure, customers can exchange or repair products within 15 days from the date of original retail purchase.

3.	12 months warranty covered from the date of original retail purchase, but the parts warranty are not covered.

4、iWiscloud iWiscloud Infrared / Multi Frequency Transmitter is warranted for the specified Warranty Period from the date of original retail purchase against defects in quality and materials under normal, non-commercial use. Abnormal usage caused damage is not warranty covered. Abnormal usage includes (force majeure), but limited to, deliberate damaging, disassembling, dismantle, manipulating warranty stick, power overloading, water damaging or other man-made caused product damages.

5. Contact Information

URL: http://en.iwiscloud.com

E-mail：service@iwiscloud.com

Yantai Wiscloud Cloud Computing Co., Ltd.  reserves its right in the terms and conditions interpretation found within this operational manual.