Nebula Module

Operating Manual

V2.0

Ⅰ. Product Features

Nebula Module integrates advantages of NORDIC radio frequency chip nRF24L01:

- Support 2.4GHz ISM frequency band, max transmitting power is 0dBm;

2． High transmission rate--2Mbps;

- Lower power consumption—current consumption is only 22uA under standby mode;

4．With 125 multi-frequency points, satisfying multi-point communication;

5．Effective communication distance: 50m

Nebula Module integrates ST company STM8 series low power consumption MCU, achieving multiple modules communicated with each other by USART setting.

Standard IO port can be connected with other devices, standard contact pin Dip2.0MM.

II. Product Specifications

| Name | Value | Unit |
| --- | --- | --- |
| Supply voltage | 2.5~3.6 | V |
| Max transmitting power | 0 | dBm |
| Max data transmission rate | 2000 | kbps |
| Current consumption（0dBm）(transmit mode) | 24.9 | mA |
| Current consumption (2000kbps) (reception mode) | 25.3 | mA |
| Temperature range | -40~+85 | ℃ |
| Data transmission rate sensitivity (1000kbps) | -85 | dBm |

III. Nebula Module Pin Introduction

| Pin | Symbol | Function |
| --- | --- | --- |
| 1 | VCC | Power |
| 2 | GND | GND |
| 3 | TX | USART output |
| 4 | RX | USART input |
| 5 | O5 | Pairing indication output |
| 6 | O4 | Power indication output |
| 7 | I3 | IO input port 3 |
| 8 | O3 | IO output port 3 |
| 9 | I2 | IO input port 2 |
| 10 | O2 | IO output port 2 |
| 11 | I1 | IO input port 1 |
| 12 | O1 | IO output port 1 |

Note: USART Baud rate 115200---data bits 8, no check bit, stop bit 1.

IV. Configuration

1 Configuration Instructions

| Frame start code | Read & write code | Command code | Data code | End mark |
| --- | --- | --- | --- | --- |
| 0XFD | 1Byte | 1Byte | Details as follows | 0XAA |

- Frame start code: The start of a frame of information.

- Read & write code: Read code: 0x00; Write code: 0x01.

- Command code

| Code | Introduction |
| --- | --- |
| 0x11 | Set up network ID |
| 0x12 | Set up Packet ID |
| 0x13 | Set up module ID |

4）Data code: Its structure is changing with the function of read/write code & function code.

（1）If read & write code is 0x00, the data code will be reserved as 0x00. Nebula module will return the current settings after instruction executed successfully, returned value as follows:

| Function | Default value | Effective value | Introduction |
| --- | --- | --- | --- |
| Network ID | 0x0000 | 0x0000~ 0xFFFE | 16 bits network ID, only with same network ID can form network automatically |
| Packet ID | 0x00 | 0x00~0xFF | The network can be divided into lots of small packets |
| Module ID | 0x0000 | 0x0000~ 0xFFFF | Communication address of module forming network |

Note: Instruction setting successful is “OK”, failed is “ERROR”

For example:

1. Set current module network ID as 0006H

FD 01 11 06 00 AA

Setting successful, returned value: OK

Setting failed, returned value: ERROR

Read current module network ID

FD 00 11 00 AA

Returned value: 06 00

Note: Network ID low-order is ahead, high-order is behind.

2.Set current module network packet ID is 04H

FD 01 12 04 AA

Setting successful, returned value: OK

Setting failed, returned value: ERROR

Read current module network packet ID

FD 00 12 00 AA

Returned value: 04

3.Set current module ID is 0009H

FD 01 13 09 00 AA

Setting successful, returned value: OK

Setting failed, returned value: ERROR

Read current module network packet ID

FD 00 13 00 AA

Returned value: 09 00

Note: Network ID low-order is ahead, high-order is behind.

- End mark AAH: The end of a frame of information.

2Data Transmission

1.Send data

Sending data to other modules through Nebula module, data format as follows:

| 0XFF | 0X05 | 0X00 | 0X55 | 0X55 | …… | 0XAA |
| --- | --- | --- | --- | --- | --- | --- |

Frame Header   Type                                             Frame Tail

Transmit mode (point to point:0x00)			Data transmitted (15 bytes)

Packet broadcast: 0x01

Network wide broadcast：0x02          Point to point: target module ID

Broadcast: 0x0000

For example:

Send data to the module (module ID is 0x001A): 0x87 0x07 0x00 0x02 0x11

Send instruction: FF 05 00 1A 00 87 07 00 02 11 00 00 00 00 00 00 00 00 00 00 00 00AA

Send data to lots of same packet module ID: 0x87 0x07x0x00 0x02 0x11

Send instruction: FF 05 01 00 00 87 07 00 02 11 00 00 00 00 00 00 00 00 00 00 00 00AA

Send data to lots of same packet module ID: 0x87 0x07x0x00 0x02 0x11

Send instruction：FF 05 02 00 00 87 07 00 02 11 00 00 00 00 00 00 00 00 00 00 00 00AA

Note: Network ID low-order is ahead, high-order is behind.

2.Receive data

Receiving data from other modules, data format as follows:

| 0XFF | 0X05 | 0X01 | 0X11 | 0X11 | …… | 0XAA |
| --- | --- | --- | --- | --- | --- | --- |

Frame Header   Type

Transmit mode（point to point:0x00）						         Frame Tail

Packet broadcast：0x01

Network broadcast：0x02） Source Module ID    Data transmitted（15 bytes）

For example:

Receive data sent by 0x002d module (point to point) : 0x87 0x07 0x00 0x02 0x11

Receive data：FF 05 00 2d 00 87 07 00 02 11 00 00 00 00 00 00 00 00 00 00 AA

Receive data sent by 0x002d module (packet broadcast): 0x87 0x07 0x00 0x02 0x11

Receive data：FF 05 01 2d 00 87 07 00 02 11 00 00 00 00 00 00 00 00 00 00 AA

Receive data sent by 0x002d module (network wide broadcast): 0x87 0x07 0x00 0x02 0x11

Receive data：FF 05 02 2d 00 87 07 00 02 11 00 00 00 00 00 00 00 00 00 00 AA

Each module needs to be set same network ID to achieve communicating with each other.

If Nebula module receive other data (not sent to it), it will transmit the data to other modules .

3 IO Instruction Settings

| Frame start code | Read & write code | Port number | Command code | End mark |
| --- | --- | --- | --- | --- |
| 0XFC | 1Byte | 1Byte | 1Byte | 0XAA |

- Frame start mode: The start of a frame of information.

- Read & write code: Read code: 0x00; write code: 0x01.

- Port number: Input/output port, the range is 1~3.

- Command code: 0x00：output low level  0x01：output high level （if the code is read code, the command code is defaulted as 0x00）

5)End mark: The end of a frame of information.

For example:

Set output port 1 output high level

FC 01 01 01 AA

Setting successful, returned value: OK

Setting failed, returned value: ERROR

Read input level of input port 2

FC 00 02 00 AA

High level, returned value: 0x01

Low level, returned value: 0x00

IO Port Operating Instruction

Users can connect button with input port to achieve iWiscloud smart switch function & pairing with smart switches. Short press button to send control signal. Long press 10s to enter into pairing mode. Long press 15s to enter into single pairing clear mode. Long press 20s to enter into all pairings clear mode. Users can also connect pairing indicator light and power indicator light. Using button to control, the output port will have level fluctuation.

URL: http://en.iwiscloud.com

E-mail：service@iwiscloud.com

Yantai Wiscloud Cloud Computing Co., Ltd.  reserves its right in the terms and conditions interpretation found within this operational manual.