iWiscloud Smart Home Control Center

IWISCLOUD SMART HOME CONTROL CENTER

Operating Manual

http://www.iwiscloud.com

【Warning】

Please do not attempt to dismantle this delicate product, willful manipulation might lead to electrical failure. Warranty will be void if product is found to be tampered with.

【Notice】：As this product constantly improve, the pictorial diagram might be an indication of the actual product delivered. Wiscloud is grateful for your choice to purchase our iWiscloud smart home control center. For your safety and maximizing our product capabilities, we would highly encourage you to study this operation manual in details. Please safe keep this operation manual for future references.

1、Product Introduction

This product had been designed and manufactured in accordance to strict International safety standards and achieved CE certifications.

- 1 Smart Home Control System

  - Interface & Icon Introduction

Diagram 1   Main Interface (Standard Edition)

1.3 Right Ports Introduction：

TF：Not Open Now;

SIM：Insert SIM Card;

LAN：Not Open Now;

USB：Not Open Now;

DC：Power Port;

2、Product Specifications

|  | Specifications |
| --- | --- |
| Name | iWiscloud Smart Home Control Center |
| Processor | Cortex-M3 72MHz |
| Working Temperature | -20℃~+70℃ |
| Display | LCD,800*480,Resistive Touch Panel |
| Wireless Devices | 315MHz，433MHz，2.4GHz，GSM/GPRS |
| Power | DC 12V |
| Ports | 1 USART, 1 SD Card, 1 SIM Card, 1 Power Port |
| Edition | Basic/Standard/Professional Edition |

3、Operating Steps

The following operating steps are for standard edition. iWiscloud Smart Home Control Center hereafter referred to as Control Center.

3.1 ON/OFF

After power on, the main interface will appear normally; Power off.

3.2 Addressing & Seeking

Addressing: The Infrared Transmitter is connected to Control Center for the first time/after resetting. The left part of icon “Wiscloud” is addressing function (the letter “W”);

Seeking: bControl Center will check if the infrared transmitter and smart sensor are connected with is successfully. The right part of icon “Wiscloud” is seeking function (the letter “d”).

Notice: Detailed pairing mode referred to infrared transmitter operation manual.

3.3 Settings

Click “Settings” and enter into.

- Security Learning: Can pair with 315Mhz security devices;

- Curtain Learning: Can pair with 433Mhz devices;

- Infrared Learning: Can pair with TV, air-conditioner infrared devices;

- Audible Alarm: When the button is “ON”, audible alarm is in working status; when the button is “OFF”, audible alarm not work anymore;

- Screen Calibration: When the button is “ON”, then click “Save” button to do screen calibration;

- Security: When the button is “ON”, enter into security status; when the button is “OFF”, no security function;

- Start Time: What time it begins;

- End Time: What time it ends;

- Superuser: Can add 3 phone numbers to receive alarm warning message; it’s necessary to insert a SIM card into smart home control center. Remote connection method referred to “12、Remote Control” introduction；

- Screen Brightness: Adjust screen brightness manually;

- Set Date: Date form is “year/month/date”;

- Set Time: Time form is “00:00:00”；

- Service Number: The number is service provider;

- Scene Binding: For 2.4G smart switch pair with scene mode of smart home control center;

- Learn Scenes: For 315Mhz remote control devices pair with scene mode of smart home control center;

- Control Scenes：For 2.4G switch, 315Mhz remote control devices;

- No Times Scenes: When the button is “ON”, all of timing function of scene modes can’t use; when the button is “OFF”, all of timing function of scene modes can work normally;

- Repeater Setup: Can pair with 315MHz、433MHz repeaters.

Note: When finishing settings, remember to click button “Save”.

3.4 Pairing

3.4.1 Infrared Devices Pairing

Pairing methods referred to infrared transmitter operation manual.

Notice: 1. For energy saving, here suggest infrared devices matched with smart sockets to use, detailed pairing steps referred to smart socket operation manual; 2. When finishing settings, remember to click button “Save”.

3.4.2 Curtain Learning

Make wireless control between iWiscloud and electric curtains.

The default setting of electric curtains remote control is 433Mhz.

Click “Settings”, Curtain Learning is “ON”, then click “Save”. Click “Controller”, then choose room and corresponded curtains, click “On”. During 5 beep sounds, then press curtain remote controller’s button “On”, the smart home control center will make one beep sound that shows the pairing has been finished. Likewise, set “Off” and “Pause”.

When curtains pairing has been finished, click Curtains Learning “OFF” in Setting interface.

Notice: When finishing settings, remember to click button “Save”.

3.4.3 315Mhz Security Devices

Make wireless control between iWiscloud and security products.

Click “Settings”, Security Learning is “ON”, Security is “OFF”, then click “Save”.

315Mhz Security Devices Learning: Click “Security”, choose the learned options, during 4 beep sounds, then triggered the security devices manually, the smart home control center will make one beep sound that shows the pairing has been finished;

Note: 1. In the security study to close the learning outside the security equipment to prevent interference with the security equipment signal. 2. When finishing settings, remember to click button “Save”.

3.4.4 Audible Alarm Learning

Power on audible alarm and smart socket, click “Controller”-“Others”, pair sockets with audible alarm, the detailed pairing method referred to smart socket’s operation manual. After pairing, click “Settings” then click “Setting”, “Audible Alarm” is “ON”; if “Audible Alarm” is “OFF”, it won’t alarm despite security alarming.

3.4.5 Sensors

Make wireless control between iWiscloud and smart sensor. The detailed operation steps referred to smart sensor operation manual.

- Users can set the thresholds by themselves.

（2） Smart Sensor Alarm

Click “Settings”-“Sensors”, according to the instruction on right column, then click “Controller” to find the room to be set. The “Max-” and “Min-” are “ON”, and “Security”-“Armed” in on, then the alarm can be triggered.

Note: When finishing settings, remember to click button “Save”.

（3）Smart sensor alarm can pair with smart home control center and other devices. The smart sensor can realize data synchronization.

Note: The smart sensor detection value is red to show it has exceeded the threshold value; if yellow, the value is normal; if green, the value is lower; if blue or 0, no connection here.

4. Other Devices Pairing Methods

（1）Smart Switch (Neutral) Pair With Smart Home Control Center

Click “Controller”, pair the switches with lamps.

Smart home control center current detection page: click “Controller”-“Lamp”-“next page”, that’s the switch current detection page.

The detailed pairing method referred to smart switch operation manual.

Note: No matter which switch the current pair with, there just display the total current through the switch, not single channel current.

（2）Smart Switch (Live) Pair With Smart Home Control Center

Click “Controller”, pair the switches with lamps.

The detailed pairing method referred to smart switch operation manual.

（3）Smart Socket Pair With Smart Home Control Center

Click “Controller”, pair the devices with power sockets.

The detailed pairing method referred to smart socket operation manual.

（4）Smart Switch Box Pair With Smart Home Control Center

The detailed pairing method referred to smart switch box operation manual.

（5）2.4G Smart Remote Controller Pair With Smart Home Control Center

2.4G Smart Remote Controller can be used as urgency button.

Click “Controller”-“Kitchen”-“Remote 1”-“Pair”,then press remote controller’s button within 15s, the pairing is finished; Click “Delete”, the paired remote controller can be deleted. When pairing is finished, then press the corresponded key of remote controller, meanwhile the smart home control center will make one beep sound and send warning message to phone. (Message notification function can be achieved by using SIM card, and also the remote connection is available. The detailed remote connection steps referred to “12、Remote Control” instruction.)

5、Partial Security Settings

When all of security devices paired successfully, choose “Settings”-“Security”; Users can ON/OFF security alarm devices.

Note: When finishing settings, remember to click button “Save”.

6、Security Interface Operation

Choose “Security” on homepage to set.

Note: “Security” is “ON” and “Security Learning” is “OFF” to achieve security alarm control.

- Choose “Partial”, then the security devices will alarm if it enters into alarm status, the closed security devices won’t alarm.

- Choose “Armed”, then the security devices will alarm if it enters into alarm status.

（3） Choose “ Disarm ”， then the opened security devices can’t alarm;

（4） Click “Cancel” to cancel the alarming devices.

When the security alarm is triggered, the remote control will be defaulted disconnected status, can restore normally after 30-40s.

7、Project

This option is dedicated to the professional staff.

Choose “Settings”-“Project”, enter password and choose “√”.

- Control Center Address Setting: To set connections between smart sensor and smart home control center;

（2） Acquisition interval / GPRS_IP address / GPRS_ port / GPRS_ID: technical staff will make configuration according to the actual condition;

（3）SMS Rechristen: Rename the message when it is “ON”;

（4）Rename by texting message must insert SIM card to achieve remote connection. Remote connection steps referred to “12、Remote Control” instruction;

Rename Message: Text a message to control center’s phone card.

Note: The punctuation “,” must be input in English.

·Rename Room: Form: cmd1, room1-room8（horizontal order）,the changed name. (the length of name no more than 4 Chinese characters; no more than 8 English letters) (Specially rename “Washroom”, no more than 2 Chinese characters; no more than 4 English letters)

Text the message example: cmd1, room1, office

Rename the room to “office”.

·Rename the lamp and switch: Form: cmd2, room1-room8（horizontal order）,the changed name. (the length of name no more than 7 Chinese characters; no more than 10 English letters)

Text the message example: cmd2, room1, top lamp, main lamp

Rename the “top lamp” to “main lamp”.

·Rename the lamp and switch: Form: cmd2, room1-room8（horizontal order）,the changed name. (the length of name no more than 7 Chinese characters; no more than 10 English letters)

Text the message example: cmd3, 1, work

Rename the “1” to “work”.

Note: When renamed one, “Project”-“SMS Rechristen” will close automatically, it’s necessary to choose again manually if rename another one.

（4）System Upgrade/Debug Mode: No need to edit;

（5）Curtain 433M: The default setting is “ON”, curtain can pair and control; Water conditioner: The default setting is “OFF”, if pairing or control it, choose “ON” and “Save”.

（6）315/433 Erasing: Choose “ON” then “Save”, to erase the paired 315MHz/433MHz devices;

（7）Switch Feedback: It can feedback the switch status to control center;

（8）Automatic Brightness Adjustment: The brightness of control center can adjust automatically according the light of smart humiture sensor;

（9）SMS On Power Up: When the home is power-on, there will push message notification. It’s necessary to insert SIM card into control center to realize message notification & remote connection. Remote connection steps referred to “12、Remote Control” instruction;

Note: When finishing settings, remember to click button “Save”.

8、Scene Mode Setting

Click “Settings”, there are 8 scene modes: Dawn, Dining, Guest, Home, Media, Sleep, Away, All off;

- Choose any of scene mode to set. There is “√” behind every device, provided that “√”, the “ON”“OFF”is valid.

- Timing Setting: Input the start time then choose “√”behind “Set Time”: . “×” is timing off; When finishing settings, remember to click button “Save”.

（3）Rename Scene Mode: Click the name of scene mode, such as “Dawn”, then there’s a pop-up box, input the changed name then choose “yes”, finally “Save”. (The length of name is no more than 5 Chinese characters; no more than 10 English letters)

（4）Choose “Settings”-“No Timed Scenes”: When it is “ON”, all of scene modes timing is invalid; when it is “OFF”, all of scene modes timing is valid. When finishing settings, remember to click button “Save”.

（5）Choose “Settings”-“Start Time”: It will be started at this time.

“End Time”：It will be ended at this time.

（6）2.4G Smart Switch Pair With Smart Home Control Center Scene Mode

Choose “Settings”-“Setting”, “Scene Binding” is “ON”, “ Control Scenes ” is “OFF”, pair scene mode with smart switch; After that, click “Settings”-“Setting”, “Scene Binding” is “OFF”, “ Control Scenes ” is “ON” to control;

（7）315Mhz Remote Control Devices Pair With Smart Home Control Center Scene Mode

Choose “Settings”-“Setting”, “Learn Scenes” is “ON”, “Control Scenes” is “OFF”, pair scene mode with 315Mhz Remote Control Devices likewise “Security Learning” pair with control center; After that, click “Settings”-“Setting”, “Learn Scenes” is “OFF”,  “Control Scenes” is “ON” to control;

Note: When finishing settings, remember to click button “Save”.

9、The Choice of Scene Mode

Choose the scene mode you want on homepage to achieve one-touch control, also can terminate the current mode.

（1）2.4G Smart Switch Control Smart Home Control Center Scene Mode

Choose “Settings”-“Setting”, “Scene Binding” is “OFF”, “Control Scenes” is “ON” to control;

（2）315Mhz Remote Control Devices Pair With Smart Home Control Center Scene Mode

Choose “Settings”-“Setting”, “Learn Scenes” is “OFF”, “Control Scenes” is “ON” to control;

10、Controller

Choose “Controller”, then pair the room or devices with smart home control center.

Every room will display temperature, humidity, VOC and CO2 values.

11、Service Interface

Users can choose goods you want to buy, then the supermarkets will provide door-to-door service in a short time. (Coming soon... ...)

12、Other Operation Instruction

（1）Smart home control center will enter into black screen mode at 9:00 pm, there is dark clock display on screen. It can restore bight when clicking center part of screen, after 3 minutes enter into black screen automatically; When the luminosity changing from bright to dark more than 3 minutes, then the smart sensor and control center will become black screen together, It can restore bight when clicking center part of screen, after 3 minutes enter into black screen automatically.

13、Remote Control

Insert a SIM card with GPRS into smart home control center when it is powered off, then repower it waiting  to , phone/tablet/computer can log in the website: ushome.iwiscloud.com input user name and password;

Background Login

- Click “Change Password”;

- Click “Smart Profile”-“Edit” to rename it, then click “Submit”;(no more than 4 Chinese characters; no more than 8 English letters)

- Click “Switch name” to rename switch, then click “Save”; (no more than 7 Chinese characters; no more than 10 English letters)

- Click “Current value”, display the instant voltage of switch on/off;

- Camera: Operate according to camera installation instruction, then copy web link and stick here; (Note: Select the camera with web support can be valid.)

- Already buy goods: Check the logistics status of goods.

- Smart sensor: Every smart sensor has value history.

Login

1、Logout: Exit the interface;

Backend: Enter into backend settings interface;

Settings: Enter into settings interface;

Home: Enter into homepage interface;

2、Room: Room list & Devices control in room;

Security: Security devices setting;

Community service: Users can choose goods you want to buy, then the supermarkets will provide door-to-door service in a short time. (Coming soon...)

Camera：After connecting with remoter camera, clicking “Camera”to check your home in anytime;

Current: Display every switch’s current status.

3、Smart sensor detection value display;

4、Smart profiles execution;

5、Specific Introduction

Click “Settings” to rename room. (no more than 4 Chinese characters; no more than 8 English letters) (Specially rename “Washroom”, no more than 2 Chinese characters; no more than 4 English letters). Click “Edit”, smart home control center synchronization; Version information and password settings.

Click “Room” to set every device in this room.

Click “Security” to set “Anti-half”, “Armed”, “Disarm”.

Community service: Users can choose goods you want to buy, then the supermarkets will provide door-to-door service in a short time. (Coming soon...)

After connecting with remoter camera, clicking “Camera”to check your home in anytime;

Click “Room”-“light” to ON/OFF the lamp;

Click “Switch” to ON/OFF appliances.

14、Smart Home Control Center Version Introduction:

iWiscloud Smart Home Control Center has three versions: Professional, Standard, and Basic versions.

Pro. Version: 8 smart profiles, 8 rooms, with “Others” option; “light” has two pages, can add 48 lamps; “switch” has two pages, can add 48 lamps; support renamed by message.

Standard version: 8 smart profiles, 8 rooms, with “Others” option; “light” has one page, can add 24 lamps; “switch” has two pages, can add 22 lamps; support renamed by message.

Basic version: 6 smart profiles, 8 rooms, without “Others” option; “light” has one page, can add 22 lamps; “switch” has one page, can add 21 lamps; not support renamed by message.

15、Notes

1、Please install the bracket in the proper position;

2、Please make sure the paired devices be installed successfully;

3、Please don’t install it at the magnetic interference places to prevent signal interference;

4、Install more than one device, the distance between each other should be more than 2cm.

16、Customer Service and Warranty

Yantai Wiscloud Cloud Computing Co., Ltd provides the following national warranty coverage for iWiscloud smart home control center:

1. If there is performance failure, customers can return, exchange or repair products within 7 days from the date of original retail purchase.

- If there is performance failure, customers can exchange or repair products within 15 days from the date of original retail purchase.

- 12 months warranty covered from the date of original retail purchase, but the parts are not covered.

- iWiscloud Smart Home Control Center is warranted for the specified Warranty Period from the date of original retail purchase against defects in quality and materials under normal, non-commercial use. Abnormal usage caused damage is not warranty covered. Abnormal usage includes (force majeure), but limited to, deliberate damaging, disassembling, dismantle, manipulating warranty stick, power overloading, water damaging or other man-made caused product damages.

17、Product Checklist

| Items | Quantity |
| --- | --- |
| iWiscloud Smart Home Control Center | 1 |
| Bracket | 1 |
| Power Adapter | 1 |
| Operating Manual | 1 |
| Warranty Card | 1 |
| Quality Control Certificate | 1 |

18、Company Information

URL: http://en.iwiscloud.com

E-mail：service@iwiscloud.com

Yantai Wiscloud Cloud Computing Co., Ltd.  reserves its right in the terms and conditions interpretation found within this operational manual.